PATEL NIDHI

Module 10: infrastructure Security an WAN Technologies

- A Cisco Catalyst switch connects to what should be individual user PCs. Each port has the same port security configuration, configured as follows: interface range gigabit Ethernet 0/1 - 24 switchport mode access switchport port-security switchport port-security mac-address sticky Which of the following answers describe the result of the port security configuration created with these commands? (Choose two)

Ans  : B. If a user connects a switch to the cable, prevents multiple            devices from sending data through the port.

D. Will allow any one device to connect to each port, but will not save     that device’s MAC address into the startup-config

2. What is the Administrative Distance of internal EIGRP routes?

Ans : B. 90

Reason : Internal EIGRP has AD = 90

- When a subnet mask is presented in binary, what do the binary 1s represent?

Ans : The network portion of an associated address

- Which switch would STP choose to become the root bridge in the selection process?

Ans : A. 32768: 11-22-33-44-55-66

Reason :Lowest Bridge ID wins, and Same priority → lowest MAC

address wins

- Which of the following devices is used by the service provider to provide WAN services?

Ans : C. WAN switch

Explanation : A. Router → Customer premises equipment (CPE)

B. Core router → Used inside ISP backbone, not for direct WAN service delivery

D. CSU/DSU → Customer-side interface device

- Your Cisco IOS router is acting as a DHCP server. Which command  will display the addresses that have been handed out to clients on the LAN?

Ans : D. show ip dhcp bindings → shows leased IPs.

- Which of the following commands would you use to enable EIGRP only on those interfaces with an IP address from 10.1.1.0 through 10.1.1.63?

Ans : A. network 10.1.1.0 0.0.0.63 ita a correct wildcard mask

- R3 has a static route configured that points toward the service provider. What command could you use to have R3 advertise an OSPFv3 default route to the internal network, regardless of whether R3 had its default static route?

Ans : E. Have R3 use the command default-information originate always in      OSPFv3 router configuration mode.

default-information originate always

Forces default route advertisement.

- You are configuring dynamic NAT on your Cisco IOS router. Which command is used to verify the interfaces that are being used as the outside interface and the inside interface?

Ans : F. show ip nat statistics  its means shows inside/outside interfaces

- When using the “show EtherChannel summary “command, what     does the “u “flag signify?

Ans : In use   u = in use (active in bundle)

- Which command could you enter to encrypt passwords?

Ans :  A. enable secret

- You are setting up a Cisco IOS router as a DHCP server. Which command is used to identify the IPv4 addresses that will be in the DHCP pool?

Ans : A. network   → defines pool address range.

- Which of the following statements are true regarding the processing

of ACLs that have been applied to router interfaces? (Choose two)

Ans :  A. Inbound ACLs will be processed before the routing table lookup  occurs
            C. Outbound ACLs will be processed after the routing table lookup has occurred

- Inbound ACL → before routing lookup and Outbound ACL → after routing lookup

- imagine you configured OSPFv2 in a small lab network. Which of the following answers list a condition that could keep the routers in your lab from learning all the routes to all the IPv4 routes in your small lab network? (Choose two)

Ans :  A. An ACL could be blocking router advertisements.

C. Any physical layer problem that would prevent two Neighbouring        routers from being able to ping each other’s IPv4 addresses in the subnet that exists between the two routers

Reason:  A ACL blocking OSPF packets and   C Physical issue → no neighbor adjacency

- Which statements describe neighbor discovery functionality in IPv6?

(Choose two)

Ans: A. Determines the link layer address of a neighbor

D. Queries for duplicate addresses

Reason :   A Finds MAC address (replaces ARP) and D Performs Duplicate Address Detection (DAD)

16. Which IPv6 prefix will the typical enterprise network receive from t                       service provider?

Ans: E. /48

Reason : Enterprises usually get /48 from ISP.

17. How should be configured a switch so that it could be accessed remotely?

Ans: C. Configure a gateway for the switch

Reason: cozz  Default gateway required for remote access.

18. Refer to the exhibit. A network technician is asked to design a small network with redundancy. The exhibit represents this design, with all hosts configured in the same VLAN. What conclusions can be made about this design?

Ans: B. Spanning-tree will need to be used.
 The design has redundant links between switches, which can create Layer 2 loops. To prevent broadcast storms and MAC table instability, Spanning Tree Protocol (STP) is required.

D. The connection between switches should be a trunk.
 Since all hosts are in the same VLAN and the VLAN must exist on both switches, the inter-switch link should be a trunk so VLAN traffic can pass correctly.

Reason :   B STP required to prevent loops and  D Switch-to-switch link should be a trunk